DESIGN AND PERFORMANCE ANALYSIS OF A PIPELINED PROCESSOR

WITH A MULTI-LEVEL CACHE MEMORY SYSTEM

CSA1221 – Computer Architecture

Report Based on the Required Design, Analytical Computation and Viva Deliverables

Analytical Processor Design Study

1. Introduction

This report presents an analytical design study of a single-core processor datapath and memory subsystem for an edge-computing workload. The design follows the assignment requirements for register transfer notation, multiple-bus organization, a 5-stage pipeline, hazard analysis, a 2-way superscalar extension, multi-level cache memory, virtual memory, AMAT, bandwidth and throughput comparison.

The proposed processor uses a 64-bit register file and datapath, a 5-stage IF-ID-EX-MEM-WB pipeline, forwarding and branch prediction for hazard mitigation, and a 2-way superscalar issue organization with out-of-order and speculative execution. The memory hierarchy contains L1, L2 and L3 caches backed by DRAM. MRAM is considered for a small non-volatile storage/cache-control level where persistence can be useful.

All numerical values in the report are design assumptions for analytical comparison. They are not measurements from physical silicon.

# 2. Deliverable Mapping

| Deliverable | Implementation / Design Area |
| --- | --- |
| 1 | Register-transfer datapath and processor design |
| 2 | ALU operations and multiple-bus organization |
| 3 | 5-stage pipeline design |
| 4 | Data and control hazard analysis |
| 5 | CPI calculation with and without mitigation |
| 6 | 2-way superscalar, out-of-order and speculative execution |
| 7 | L1/L2/L3 cache hierarchy and virtual memory |
| 8 | AMAT and memory-system performance analysis |
| 9 | Bandwidth, throughput comparison, reflection and SDG 9 |
| 10 | Complete end-to-end design demonstration and conclusion |

# Deliverable 1: Register-Transfer Datapath and Processor Design

## Purpose

The datapath defines how instructions move operands between the program counter, register file, ALU, cache and memory. A multiple-bus arrangement is used so that two source operands can be read and supplied to the ALU without a single-bus bottleneck.

## Pseudocode

START
    Initialize PC and 32 × 64-bit register file
    FETCH instruction from instruction cache using PC
    DECODE opcode and identify source/destination registers
    READ source operands through Bus A and Bus B
    EXECUTE ALU operation or calculate memory address
    IF load/store
        ACCESS data cache
    IF instruction writes a register
        WRITE result through Bus C
    UPDATE PC
STOP

## Sample Input

| Instruction Set | LOAD, STORE, ADD, SUB, BEQ |
| --- | --- |
| Register Width | 64-bit |
| Registers | 32 |
| Bus Organization | Bus A, Bus B, Bus C |

## Sample Output

REGISTER-TRANSFER / MULTI-BUS DATAPATH

## Result / Observation

The proposed datapath provides two operand-read paths and one result-write path. This supports ALU operations and memory-address calculation with fewer bus conflicts than a single-bus organization.

# Deliverable 2: ALU Operations and Multiple-Bus Organization

## Purpose

Register-transfer notation specifies the movement of data for the required instruction set. The ALU performs arithmetic, address-generation and branch comparison operations.

## Pseudocode

LOAD R1, [R2 + imm] : MAR ← R2 + imm ; R1 ← M[MAR]
STORE R1, [R2 + imm]: MAR ← R2 + imm ; M[MAR] ← R1
ADD R3, R1, R4       : R3 ← R1 + R4
SUB R5, R3, R6       : R5 ← R3 - R6
BEQ R5, R0, L        : IF R5 = R0 THEN PC ← L

## Sample Input

| Example | ADD R3, R1, R4 |
| --- | --- |
| Bus A | R1 → ALU input A |
| Bus B | R4 → ALU input B |
| ALU | ADD |
| Bus C | ALU result → R3 |

## Sample Output

R1 and R4 are transferred simultaneously on Bus A and Bus B; the ALU produces R1 + R4; Bus C writes the result into R3.

## Result / Observation

The RTN and bus scheme clearly models inter-unit data movement and supports the required LOAD, STORE, ADD and branch operations.

# Deliverable 3: 5-Stage Pipeline Design

## Purpose

The scalar processor is divided into five stages: Instruction Fetch (IF), Instruction Decode/Register Read (ID), Execute (EX), Memory Access (MEM), and Write Back (WB).

## Pseudocode

FOR each clock cycle
    IF  : fetch instruction and update fetch PC
    ID  : decode instruction and read registers
    EX  : perform ALU operation / address / branch comparison
    MEM : access data cache for LOAD or STORE
    WB  : write result into destination register
END FOR

## Sample Input

| Pipeline | IF → ID → EX → MEM → WB |
| --- | --- |
| Clock | 2 GHz |
| Issue Width | 1 for scalar baseline |

## Sample Output

## Result / Observation

The 5-stage pipeline allows multiple instructions to occupy different stages at the same time, increasing instruction throughput compared with a non-pipelined design.

# Deliverable 4: Data and Instruction Hazard Analysis

## Purpose

The selected instruction sequence contains RAW data dependencies and a conditional branch. These create data and control hazards that must be handled to maintain correct execution.

## Pseudocode

Sequence:
I1: LW  R1, 0(R2)
I2: ADD R3, R1, R4
I3: SUB R5, R3, R6
I4: BEQ R5, R0, TARGET
I5: ADD R7, R7, R8

FOR each dependency
    IF producer result is unavailable
        INSERT stall OR forward result
FOR branch
    PREDICT direction
    IF prediction is wrong
        FLUSH younger instructions and restart at target

## Sample Input

| Data Hazard 1 | I1 → I2 (load-use RAW) |
| --- | --- |
| Data Hazard 2 | I2 → I3 (ALU RAW) |
| Control Hazard | I4 conditional branch |

## Sample Output

Without forwarding, dependent instructions wait for register write-back. With forwarding, the ALU dependency can be resolved directly; the load-use dependency still requires one stall in the assumed 5-stage pipeline. Branch prediction reduces control-hazard delay when the prediction is correct.

## Result / Observation

At least two data hazards and one control hazard are explicitly identified and resolved using stalling, forwarding and prediction.

# Deliverable 5: CPI Calculation With and Without Hazard Mitigation

## Purpose

CPI is compared for the same 12-instruction workload. The baseline assumes no forwarding and a two-cycle dependency penalty for each of two RAW hazards, plus a two-cycle branch penalty. The mitigated design uses forwarding, one stall for the load-use dependency, and a correctly predicted branch.

## Pseudocode

N ← 12
Ideal pipeline cycles ← N + 4
Unmitigated penalty ← 2 RAW × 2 + 1 branch × 2 = 6
Mitigated penalty ← 1 load-use stall + 0 branch penalty = 1
CPI ← (Ideal cycles + penalty) / N

## Sample Input

| Instructions | 12 |
| --- | --- |
| Ideal pipeline cycles | 12 + 4 = 16 |
| Unmitigated extra cycles | 6 |
| Mitigated extra cycles | 1 |

## Sample Output

Without mitigation: Total cycles = 16 + 6 = 22; CPI = 22 / 12 = 1.83.

With forwarding/stall/prediction: Total cycles = 16 + 1 = 17; CPI = 17 / 12 = 1.42.

## Result / Observation

Hazard mitigation reduces the estimated CPI from approximately 1.83 to 1.42, demonstrating the performance value of forwarding and branch prediction.

# Deliverable 6: 2-Way Superscalar, Out-of-Order and Speculative Execution

## Purpose

The scalar pipeline is extended into a 2-way superscalar processor capable of dispatching up to two independent instructions per cycle. An issue queue, reservation stations, reorder buffer and separate execution units support out-of-order and speculative execution.

## Pseudocode

FETCH up to 2 instructions
DECODE and RENAME registers
PLACE ready instructions in issue queue
WHILE execution units are available
    SELECT ready instructions, possibly out of program order
    EXECUTE on ALU0, ALU1 or Load/Store unit
    WRITE results to waiting consumers
    RETIRE instructions in program order using ROB
IF branch prediction is wrong
    FLUSH younger ROB entries
    RESTORE rename state
    FETCH from correct target

## Sample Input

| Issue Width | 2 instructions/cycle |
| --- | --- |
| Execution Units | ALU0, ALU1, Load/Store |
| Scheduling | Out-of-order |
| Recovery | ROB flush + rename-state recovery |

## Sample Output

## Result / Observation

The proposed superscalar design increases instruction-level parallelism while the reorder buffer preserves precise architectural state and enables recovery after branch misprediction.

# Deliverable 7: L1/L2/L3 Cache Hierarchy and Virtual Memory

## Purpose

The memory subsystem is designed to reduce average access latency while providing sufficient capacity for an edge-computing workload.

## Pseudocode

L1: small, fast, private cache
    IF hit → return data
    ELSE access L2
L2: larger cache
    IF hit → return data to L1
    ELSE access L3
L3: shared last-level cache
    IF hit → return data
    ELSE access DRAM
Virtual memory:
    Translate virtual page number using TLB
    IF TLB miss → page-table walk
    IF page not resident → page fault and fetch from storage

## Sample Input

| L1 Cache | 32 KB instruction + 32 KB data, 4-way, LRU |
| --- | --- |
| L2 Cache | 512 KB, 8-way, LRU |
| L3 Cache | 8 MB, 16-way, LRU |
| DRAM | Main memory, 80 ns assumed access |
| Page Size | 4 KB |
| TLB | 64-entry, 4-way |
| Non-volatile choice | MRAM for a small persistent cache/control level |

## Sample Output

The hierarchy is L1 → L2 → L3 → DRAM. LRU is used as the replacement policy because it is simple and effective for the assumed workload. Virtual memory uses 4 KB pages, with the TLB reducing repeated page-translation cost.

## Result / Observation

The design satisfies the required cache-level, associativity, replacement-policy and paging decisions while balancing latency, capacity and implementation complexity.

# Deliverable 8: AMAT and Memory-System Performance Analysis

## Purpose

Average Memory Access Time is calculated for the proposed 3-level hierarchy and compared with a simpler 2-level design.

## Pseudocode

AMAT = L1_hit_time
       + L1_miss_rate × (L2_hit_time
       + L2_miss_rate × (L3_hit_time
       + L3_miss_rate × DRAM_time))

## Sample Input

| L1 hit time / hit rate | 1 ns / 95% |
| --- | --- |
| L2 hit time / hit rate | 10 ns / 90% |
| L3 hit time / hit rate | 30 ns / 90% |
| DRAM penalty | 80 ns |
| Alternative A | 2-level: L1 + L2 + DRAM |

## Sample Output

Alternative A, 2-level AMAT = 1 + 0.05 × (10 + 80) = 5.50 ns.

Proposed 3-level AMAT = 1 + 0.05 × [10 + 0.10 × (30 + 80)] = 2.05 ns.

The additional L3 cache reduces AMAT by approximately 62.7% compared with the 2-level alternative.

## Result / Observation

The 3-level cache hierarchy provides substantially lower average memory access time under the assumed hit rates.

# Deliverable 9: Bandwidth, Throughput, Reflection and SDG 9 Relevance

## Purpose

This deliverable compares scalar and 2-way superscalar performance and explains the design trade-offs and learning outcomes.

## Pseudocode

Throughput (instructions/s) = Clock frequency / CPI
IPC = Issue width / CPI
Peak L1 interface bandwidth = Issue width × 8 bytes × Clock frequency
Compare:
    Alternative A: scalar, 5-stage
    Alternative B: 2-way superscalar, OOO/speculative

## Sample Input

| Clock | 2 GHz |
| --- | --- |
| Scalar mitigated CPI | 1.42 |
| Superscalar assumed CPI | 0.75 |
| Data path width | 64-bit = 8 bytes |
| Issue widths | 1-way vs 2-way |

## Sample Output

Scalar throughput ≈ 2 GHz / 1.42 = 1.41 billion instructions/s.

2-way superscalar throughput ≈ 2 GHz / 0.75 = 2.67 billion instructions/s.

Theoretical L1 data bandwidth is 16 GB/s for a 1-way 64-bit path and 32 GB/s for a 2-way path, assuming one 64-bit transfer per issued instruction.

## Design Reflection

The major design trade-off is between complexity and performance. A 3-level cache adds area and power but significantly lowers AMAT. Forwarding and prediction require additional control logic but reduce pipeline stalls. Superscalar out-of-order execution improves instruction-level parallelism, while the reorder buffer and recovery logic increase implementation complexity.

A key implementation challenge is keeping the assumptions consistent across CPI, cache-hit rates and bandwidth calculations. The exercise demonstrates how datapath organization, pipeline hazards and memory latency interact rather than acting as isolated design problems.

## SDG 9 Mapping

SDG 9 – Industry, Innovation and Infrastructure: The design supports the development of efficient and resilient computing infrastructure by improving processor throughput, reducing memory latency and evaluating architecture alternatives before hardware implementation.

## Result / Observation

The 2-way superscalar design provides the higher analytical throughput, while the 3-level memory hierarchy provides the lower AMAT. The combined design is therefore selected as the preferred edge-computing architecture.

# Deliverable 10: Complete End-to-End Design Demonstration

## Purpose

The final demonstration connects all design decisions into one processor workflow, from instruction fetch through execution and memory access to retirement and performance evaluation.

## Pseudocode

STEP 1  Initialize 64-bit register file and multi-bus datapath
STEP 2  Fetch and decode instructions
STEP 3  Execute using 5 pipeline stages
STEP 4  Detect RAW and control hazards
STEP 5  Apply forwarding, stall and branch prediction
STEP 6  Dispatch up to 2 instructions in superscalar mode
STEP 7  Schedule ready instructions out of order
STEP 8  Retire in order using the reorder buffer
STEP 9  Access L1 → L2 → L3 → DRAM when required
STEP 10 Calculate CPI, AMAT, bandwidth and throughput
STEP 11 Compare alternatives and select preferred design
END

## Sample Input

| Workload | 12-instruction dependency/branch sequence |
| --- | --- |
| Architecture | 2-way superscalar, 5-stage pipeline |
| Memory | L1 + L2 + L3 + DRAM |
| Evaluation | CPI, AMAT, bandwidth, throughput |

## Sample Output

STEP 1  Datapath and multi-bus organization defined.

STEP 2  Five pipeline stages established.

STEP 3  Two RAW hazards and one control hazard identified.

STEP 4  Forwarding, one load-use stall and branch prediction applied.

STEP 5  2-way out-of-order/speculative organization established.

STEP 6  L1/L2/L3 hierarchy and virtual memory integrated.

STEP 7  3-level AMAT calculated as 2.05 ns.

STEP 8  Scalar throughput ≈ 1.41 GIPS; superscalar estimate ≈ 2.67 GIPS.

END-TO-END PROCESSOR DESIGN ANALYSIS COMPLETED SUCCESSFULLY

## Result / Observation

All required analytical components are integrated into a consistent processor design. The selected architecture combines a multiple-bus datapath, hazard-aware pipeline, 2-way superscalar execution and a 3-level cache hierarchy.

# Overall Conclusion

The proposed processor design satisfies the assignment requirements by modelling register transfers and multiple-bus data movement, designing a 5-stage pipeline, identifying and resolving data and control hazards, calculating CPI, extending the design to a 2-way superscalar processor, and supporting it with L1, L2 and L3 caches plus virtual memory.

Under the stated assumptions, the mitigated scalar pipeline achieves an estimated CPI of 1.42, while the 2-way superscalar design is estimated at 2.67 billion instructions/s. The 3-level cache hierarchy achieves an AMAT of 2.05 ns compared with 5.50 ns for the 2-level alternative. These results justify the proposed combination of superscalar execution and multi-level caching for an edge-computing processor.

The numerical results are analytical estimates based on explicitly stated assumptions. Actual silicon performance would depend on workload behavior, technology node, cache implementation, branch accuracy, memory controller design, power limits and many other microarchitectural factors.